🛡️ Cisco Layer 2 Switch Hardening Template

1. Layer 2 Security (The "Port" Level)

This is the most important part for a switch. It prevents "MAC Flooding" and "Man-in-the-Middle" attacks.

! --- Port Security: Prevent unauthorized devices ---
interface range e0/0 - 3
 switchport mode access
 switchport port-security
 switchport port-security maximum 2          ! Allow only 2 MAC addresses per port 1pc 1 ip phone
 switchport port-security violation shutdown ! Kill the port if a 3rd device connects
 switchport port-security aging time 2
 spanning-tree portfast                      ! Speed up connection for PCs
 spanning-tree bpduguard enable               ! Prevent loops if a user plugs in a mini-switch

2. VLAN Hardening (The "Data" Level)

Never leave unused ports in the default "VLAN 1." This is where "VLAN Hopping" attacks happen.

Bash

! --- Create a "Black Hole" VLAN for unused ports ---
vlan 666
 name UNUSED_PORTS
exit

! --- Move all unused ports to the Black Hole and SHUT them ---
interface range e1/0 - 3
 switchport access vlan 666
 switchport mode access
 shutdown

3. Anti-Spoofing & MITM Protection

In your Cybersecurity studies, you’ve likely heard of DHCP Snooping and ARP Inspection. These prevent a hacker from pretending to be the "Default Gateway."

Bash

! --- DHCP Snooping: Only allow DHCP from the Server port ---
ip dhcp snooping
ip dhcp snooping vlan 10,20,99

interface e0/0 (Port facing your Windows Server/Router)
 ip dhcp snooping trust

! --- Dynamic ARP Inspection (DAI): Prevents ARP Poisoning ---
ip arp inspection vlan 10,20,99
interface e0/0
 ip arp inspection trust

4. Global Security Settings

Bash

! --- Disable insecure protocols ---
no ip http server
no ip http secure-server
no service finger
no service udp-small-servers
no service tcp-small-servers

! --- Password Encryption ---
service password-encryption
enable secret YourStrongEnablePassword